Online UPS household energy storage   power supply product specification

ES-10000A/B/C

- 1、Overview:

- ES-10000 series, our company's latest online UPS household energy storage power supply, adopts sheet metal baking process, lightweight and portable,

- stylish and elegant appearance, using power type lithium iron phosphate batteries as energy storage to provide continuous power for the host.

- This product is a dedicated online UPS household energy storage power supply designed for areas with power shortages, unstable grid voltage, frequent power outages, and high precision equipment requirements.

- The product is designed with ultra wide voltage input and automatic voltage regulation output to ensure stable load operation.  The product is designed for zero second switching, ensuring that the load will not shut down due to power outages during use.  Can meet 100% of the power outage needs of all electrical equipment!  The product comes with a stable frequency function, which ensures that electrical appliances will not be burned out due to low or high frequency when carrying inductive loads.

- This design, applied in energy storage systems, can solve the problems of unstable grid voltage, frequent power outages, and easy damage to electrical appliances during household use.  Both mains power and solar energy charge the energy storage simultaneously, ensuring that the equipment can operate for a long time.  Solved the problem of electricity consumption in people's daily life, it is a product that truly benefits humanity and is also a necessary product!

- 2、 Appearance display:

- 3、Function display:

Front Panel Control

Rear panel output

AC inverter output touch switch

System startup touch switch

7-inch display screen of the device

DC output tactile switch

Inverter AC output socket 16A

Inverter AC output socket 16A

Four USB-A ports with 18W independent output

Two PD60W independent outputs

Two parallel 12V 100W outputs

Two way solar input interface GX-20

Communication high-power input interface 50A

Inverter AC high-power output interface 50A

Inverter AC output socket 32A

RS485 communication output

- 4、Power parameters:

| - Name | - Parameter | - Unit | - Product model |  |  |
| --- | --- | --- | --- | --- | --- |
| - AC Input |  |  | - ES-10000A | - ES-10000B | - ES-10000C |
|  | - Input Voltage | - V/AC | - AC85V-310V | - AC85V-310V | - AC85V-310V |
|  | - Input frequency | - HZ | - 30-100HZ | - 30-100HZ | - 30-100HZ |
|  | Input voltage to efficiency ratio | - AC/85V | - 75% | - 75% | - 75% |
|  |  | - AC/95V | - 85% | - 85% | - 85% |
|  |  | - AC/110V | - 90% | - 90% | - 90% |
|  |  | - AC/130V | - 95% | - 95% | - 95% |
|  |  | - AC/310V | - 98% | - 98% | - 98% |
|  | - Input power factor | - 1 | - 0.99 | - 0.99 | - 0.99 |
|  | - Power on mode |  | - Soft start mode | - Soft start mode | - Soft start mode |
|  | - Under voltage protect | - AC | - 80V | - 80V | - 80V |
|  | - Over voltage protect | - AC | - 320V | - 320V | - 320V |
|  | - Input temperature Protection | - ℃ | - 85℃ | - 85℃ | - 85℃ |
|  | - Input overload protection | - W | - 12800W | - 12800W | - 12800W |
| - AC Charging | - Charging input voltage range | - V/AC | - AC85V-310V | - AC85V-310V | - AC85V-310V |
|  | - Charging input frequency | - HZ | - 30-100HZ | - 30-100HZ | - 30-100HZ |
|  | - Charge efficiency | - % | - 95% | - 95% | - 95% |
|  | - charging power | - W | - 2800W | - 2800W | - 2800W |
|  | - Charging Voltage | - V | - 55.2V | - 55.2V | - 55.2V |
|  | - Charging current | - A | - 50A | - 50A | - 50A |
|  | - Charging method | - CC/CV | - Constant current first, then constant voltage | - Constant current first, then constant voltage | - Constant current first, then constant voltage |
| - Solar Charging | - Input Voltage | - DC/V | - DC/60-100V | - DC/60-100V | - DC/60-100V |
|  | - Charging method |  | - MPPT | - MPPT | - MPPT |
|  | - Input maximum power | - W | - 2400W | - 2400W | - 2400W |
|  | - Charge efficiency | - % | - 98% | - 98% | - 98% |
|  | - Overvoltage protection | - DC/V | - 150V | - 150V | - 150V |
|  | - Under voltage protection | - DC/V | - 55V | - 55V | - 55V |
| - On line - AC mode output | - Standard output voltage | - AC/V | - 220V | - 220V | - 220V |
|  | - No-load output voltage | - AC/V | - 228V | - 228V | - 228V |
|  | - Load output voltage | - AC/V | - Between 210V-220V | - Between 210V-220V | - Between 210V-220V |
|  | - Output frequency | - HZ | - 50HZ | - 50HZ | - 50HZ |
|  | - Output frequency fluctuation range | - HZ | - 48-52HZ | - 48-52HZ | - 48-52HZ |
|  | - Output power | - W | - 8000W | - 8000W | - 8000W |
|  | - Over load protection | - W | - 11KW | - 11KW | - 11KW |
|  | - Output efficiency | - % | - 98% | - 98% | - 98% |
|  | - Switch mode |  | - Touch switch (soft start mode) | - Touch switch (soft start mode) | - Touch switch (soft start mode) |
|  | - Output waveform |  | - AC pure sine wave | - AC pure sine wave | - AC pure sine wave |
|  | - Temperature protection | - ℃ | - 85℃ | - 85℃ | - 85℃ |

| - Name | - Parameter - With grid power load power source | - Unit | - Product model |  |  |
| --- | --- | --- | --- | --- | --- |
| - Online switching mode |  |  | - ES-10000A | - ES-10000B | - ES-10000C |
|  |  |  | - Originating from AC power supply | - Originating from AC power supply | - Originating from AC power supply |
|  | - Without grid power load power source |  | - Originating from battery inverters | - Originating - from battery inverters | - Originating from battery inverters |
|  | - With grid power, load power supply after power outage |  | - Battery inverter switches power supply in 0 seconds | - Battery inverter switches power supply in 0 seconds | - Battery inverter - switches power supply in 0 seconds |
|  | - No grid power, load power supply after power outage |  | - Switching power supply from the mains in 0 seconds | - Switching - power supply from the mains in 0 seconds | - Switching power supply from the mains in 0 seconds |
|  | - Switching time |  | - 0 seconds | - 0 seconds | - 0 seconds |
|  | - Voltage fluctuation range | - V | - ±5% | - ±5% | - ±5% |
|  | - Frequency fluctuation range | - HZ | - 48-52HZ | - 48-52HZ | - 48-52HZ |
| - Offline inverter output | - Standard output voltage | - AC/V | - 220V | - 220V | - 220V |
|  | - No-load output voltage | - AC/V | - 228V | - 228V | - 228V |
|  | - Load output voltage | - AC/V | - Between 210V-220V | - Between 210V-220V | - Between 210V-220V |
|  | - Output frequency | - HZ | - 50HZ | - 50HZ | - 50HZ |
|  | - Output frequency fluctuation range | - HZ | - 48-52HZ | - 48-52HZ | - 48-52HZ |
|  | - Output power | - W | - 10KW | - 10KW | - 10KW |
|  | - Over load protection | - W | - 11KW | - 11KW | - 11KW |
|  | - Output average efficiency | - % | - 92% | - 92% | - 92% |
|  | - Output overload protection time | - ms | - 50 milliseconds | - 50 milliseconds | - 50 milliseconds |
|  | - Short Circuit Protection |  | - Yes | - Yes | - Yes |
|  | - Switch mode |  | - Touch switch (soft start mode) | - Touch switch (soft start mode) | - Touch switch (soft start mode) |
|  | - Output waveform |  | - AC pure sine wave | - AC pure sine wave | - AC pure sine wave |
| - Temperature | - Output temperature protection | - ℃ | - 85℃ | - 85℃ | - 85℃ |
|  | - Fan start temperature | - ℃ | - 45℃ | - 45℃ | - 45℃ |
| - DC - 12V Output - Voltage | - Standard output voltage | - V | - 12V/DC | - 12V/DC | - 12V/DC |
|  | - Output voltage range | - V | - 12V-12.8V | - 12V-12.8V | - 12V-12.8V |
|  | - Output overcurrent protection | - A | - Automatic recovery with/without load | - Automatic recovery with/without load | - Automatic recovery with/without load |
|  | - Short Circuit Protection |  | - Automatic recovery with/without load | - Automatic recovery with/without load | - Automatic recovery with/without load |
|  | - Output power | - W | - 100W | - 100W | - 100W |
|  | - Output two channels of DC |  | - Two shared routes | - Two shared routes | - Two shared routes |
| - USB Output | - Output voltage | - V | - 5V/9V/12V | - 5V/9V/12V | - 5V/9V/12V |
|  | - Maximum output power | - W | - 24W | - 24W | - 24W |
|  | - Protocol method |  | - QC3.0 standard | - QC3.0 standard | - QC3.0 standard |
|  | - Short circuit protection function |  | - Automatic recovery with/without load | - Automatic recovery with/without load | - Automatic recovery with/without load |
|  | - Output four channels |  | - Four independent outputs | - Four independent outputs | - Four independent outputs |
| - Type Output | - Output voltage | - V | - 5V/9V/12V/15V/20V | - 5V/9V/12V | - 5V/9V/12V |
|  | - Maximum output power | - W | - 60W | - 60W | - 60W |
|  | - Protocol method |  | - PD protocol standard | - PD protocol standard | - PD protocol standard |
|  | - Short circuit protection function |  | - Automatic recovery with/without load | - Automatic recovery with/without load | - Automatic recovery with/without load |
|  | - Output four channels |  | - Four independent outputs | - Four independent outputs | - Four independent outputs |

| - Name | - Parameter | - Unit | - Product model |  |  |
| --- | --- | --- | --- | --- | --- |
| - Parameters of lithium iron battery pack | - Charging time of lithium iron battery | - H | - 90% in 4 hours/100% in 5 hours | - 90% in 5 hours/100% in 6 hours | - 7 hours 80%/8 hours 100% |
|  | - Rated voltage | - V/DC | - 51.2V | - 51.2V | - 51.2V |
|  | - Voltage range | - V/DC | - 44.8V-55.2V | - 44.8V-55.2V | - 44.8V-55.2V |
|  | - Type |  | - Lithium iron phosphate square cell - aluminum shell | - Lithium iron phosphate square cell - aluminum shell | - Lithium iron phosphate square cell - aluminum shell |
|  | - Cycle life |  | - 4500 times ≥ 80% capacity | - 4500 times ≥ 80% capacity | - 4500 times ≥ 80% capacity |
|  | - Battery model and capacity |  | - 50160118/100AH | - 50160118/100AH | - 50160118/100AH |
|  | - Composition |  | - 16 strings in 2 parallel | - 16 strings in 4 parallel | - 16 strings and 8 parallel |
|  | - Total capacity |  | - 10240WH (10.24 kWh) | - 20480WH (20.48 kWh) | - 30720WH (30.72 kWh) |
|  | - Protection method |  | - BMS hardware+software protection | - BMS hardware+software protection | - BMS hardware+software protection |
|  | - Product communication method |  | - RS485 protocol | - RS485 protocol | - RS485 protocol |
| - Product size and weight | - Length | - CM | - 80 | - 80 | - 80 |
|  | - Product width | - CM | - 34 | - 34 | - 34 |
|  | - Product height | - CM | - 65 | - 95 | - 127 |
|  | - Net weight | - KG | - 100 | - 169 | - 238 |
|  | - Size after packaging | - CM | - 80*34*65 | - 80*34*95 | - 80*34*127 |
|  | - Weight after packaging |  | - 110 | - 179 | - 248 |

- 5、Functional principle diagram:

- 6、PCB wiring diagram:

- Boost section, Inverter section, UPS switching, Charging control

- 7、Function panel operation instructions:

- （1）The "AC INPUT" character indicates the presence of AC mains power input, while the "SOL CHARGE" character indicates the presence of solar charging. When solar charging occurs, the characters light up, and the input power character on the display panel constantly shows the solar charging power.

- （2）The temperature display character (OVERHEATING) indicates that during the charging or discharging process, the internal temperature of the battery pack exceeds 55 degrees. When the temperature exceeds 65 degrees, the battery will not output to the outside world and the machine will shut down.

- （3）The fan character (FAN RUN) indicates that the temperature of the heat sink on the motherboard has exceeded 45 degrees and the fan is automatically running.

- （4）The mobile phone charging character (USB/TYPE-C) indicates that the USB and TYPE-C circuits are working properly. It is possible that mobile products such as phones are being charged, and during use, the output power on the panel will constantly display the current discharge power.

- （5）When the DC discharge character (12VDC OUT) lights up, it indicates that the 12V output is normal and the 12V output function can be used. When there is a load, the output power on the panel will constantly display the current discharge power.

- （6）AC OUTPUT indicates that when there is an inverter output or AC output, the character will light up. When there is an inverter output, the character will light up. When there is a load, the current discharge power will be displayed on the display panel at all times;  When there is a mains input, the display panel only shows the charging power and not the discharging power. At the same time, the character (UPS POWER) lights up, indicating that the UPS power is enabled when there is mains input. The load output uses the power of the input mains, so the discharging power will not be displayed on the display panel.

- （7）The battery character on the display panel displays the current battery level and percentage in real-time. If it is only a host, it will only display the current battery level of one battery pack on the host.  After adding a battery pack, we calculate the power of two batteries and then perform an average algorithm to display the average power of the two batteries;  After adding two battery packs, we calculate the power of the three batteries and then perform an average algorithm to display the average power of the three batteries.

- （8）To turn on the power, press and hold for 3 seconds until the display screen lights up. At this time, you can release the power button. After turning on, the current battery level and other parameters will not be displayed immediately. It will wait for 3 seconds before displaying normally because communication data will be delayed.

- （9）When using a mobile phone for DC charging, press and hold for 3 seconds to activate the DC and mobile phone charging functions.

- （10）Inverter AC output input is required. After pressing the AC power button for 3 seconds, the display screen will not immediately show the inverter output character. It is necessary to wait for 3 seconds for the character to appear because the device's startup AC output is a soft start process, and the AC inverter voltage first changes from low to high to a certain value before stabilizing.

- 8、Packing list

| Serial Number | name | quantity |
| --- | --- | --- |
| 1 | Power host | 1 |
| 2 | Communication input adapter cable | 1 |
| 3 | Communication output adapter cable | 1 |
| 3 | Output line DC5.5 * 2.1 | 1 |
| 4 | Solar input line (GX-20) | 1 |
| 5 | Instruction manual, warranty card, certificate of conformity | 1 |

ES-10000A

ES-10000B

ES-10000C